Karel Programs

IPL_ROBOTIQ_GRIPPER_MOVE

IPL_ROBOTIQ_GRIPPER_ACT

IPL_ROBOTIQ_GRIPPER_GRIP_CHECK

TP Programs

IPL_ROBOTIQ_GRIPPER_OPEN

IPL_ROBOTIQ_GRIPPER_CLOSE

Routines

move_gripper

get_gripper_position

is_gripper_activated

get_obj_detection_code

get_gripper_fault

# Karel Programs

## IPL_ROBOTIQ_GRIPPER_MOVE

Disclaimer: This Karel program requires parameters from a TP program to function properly.
                    Use the “move_gripper” routine instead if you don’t need to go through a TP program.

(slave_id : INTEGER; grip_pose : INTEGER; speed : INTEGER; force : INTEGER; wait_motion : INTEGER)

- slave_id : 9, 2, 3 or 4

- grip_pose : between 0 and 255

- speed : between 0 and 255

- force : between 0 and 255

- wait_motion : 0 or 1  (Will complete the motion before the next command.)

## IPL_ROBOTIQ_GRIPPER_ACT

(slave_id_from_set_var : INTEGER; slave_id : INTEGER)

- slave_id_from_set_var : 0 or 1

  - 0: slave_id will be provided by TP program

  - 1: slave_id will be provided by SET_VAR through 'rqSlaveId'

- slave_id : 9, 2, 3 or 4

Here is the way to call this program from a karel file:

SET_VAR(entry, 'IPL_ROBOTIQ_GRIPPER_ACT', 'rq_live_test', slave_id_from_set_var, status)

SET_VAR(entry, 'IPL_ROBOTIQ_GRIPPER_ACT', 'rqSlaveId', slave_id, status)

CALL_PROG('IPL_ROBOTIQ_GRIPPER_ACT', prog_index)

## IPL_ROBOTIQ_GRIPPER_GRIP_CHECK

Disclaimer: This Karel program requires parameters from a TP program to function properly.
                    Use the “get_obj_detection_code” routine instead if you don’t need to go through a TP program.

(register_id : INTEGER; slave_id : INTEGER)

- register_id : id of a FANUC registers R[ ] to share and keep value.

- slave_id : 9, 2, 3 or 4

# TP Programs

## IPL_ROBOTIQ_GRIPPER_OPEN

- Calls IPL_ROBOTIQ_GRIPPER_MOVE karel program with provided parameters to open the gripper. Default position value is 0.

## IPL_ROBOTIQ_GRIPPER_CLOSE

- Calls IPL_ROBOTIQ_GRIPPER_MOVE karel program with provided parameters to close the gripper. Default position value is 255.

# Routines

Routines can be used in Karel programs.

## move_gripper

(slave_id : INTEGER; grip_pose : INTEGER; speed : INTEGER; force : INTEGER; wait_motion : INTEGER) : BOOLEAN FROM IPL_ROBOTIQ_GRIPPER_MOVE

- slave_id : 9, 2, 3 or 4

- grip_pose : between 0 and 255

- speed : between 0 and 255

- force : between 0 and 255

- wait_motion : 0 or 1  (Will complete the motion before the next command.)

- Returns TRUE when completed.

## get_gripper_position

(slave_id : INTEGER) : INTEGER FROM IPL_ROBOTIQ_GRIPPER_GET_POSITION

- slave_id : 9, 2, 3 or 4

- Returns the actual gripper position between 0 and 255.

## is_gripper_activated

(slave_id : INTEGER) : BOOLEAN FROM IPL_ROBOTIQ_GRIPPER_ACT

- slave_id : 9, 2, 3 or 4

- Returns TRUE or FALSE depending on the activation state.

## get_obj_detection_code

(slave_id : INTEGER) : INTEGER FROM IPL_ROBOTIQ_GRIPPER_GRIP_CHECK

- slave_id : 9, 2, 3 or 4

- Returns:

  - 0 = Object unknown

  - 1 = Object detected on open

  - 2 = Object detected on close

  - 3 = No object detected

## get_gripper_fault

(slave_id : INTEGER) : INTEGER FROM IPL_ROBOTIQ_GRIPPER_GET_FAULT

- slave_id : 9, 2, 3 or 4

- Returns:

  - 0 = NONE

  - 3 = BRAKE_PROBLEM

  - 4 = BLOCKED_AT_START

  - 5 = NOT_ACT

  - 7 = GTO_WHILE_NOT_ACT

  - 8 = OVER_TEMPERATURE

  - 9 = COM_NOT_READY

  - 10 = UNDERVOLTAGE

  - 11 = ATR_IN_PROGRESS

  - 12 = INTERNAL

  - 13 = INIT

  - 14 = OVERCURRENT

  - 15 = ATR_COMPLETED